Imaging Quality Assurance Programme

AAC 10: There is an established quality assurance programme for imaging services

- Policy:

Quality Improvement Program Applicable for radiology

Quality Improvement program of radiology is focused on following key characteristics.  These key characteristics shall meet the specified acceptance criteria / norms and are monitored on weekly basis to ensure that they meet the acceptance criteria.

A record for this is maintained in quality assurance register of the department.

- Purpose

To document quality improvement programme

To verify and validate imaging results

For surveillance of imaging results

To take corrective and preventive actions

- Definition

Absorbed Dose- The amount of energy imparted to matter by ionizing radiation per unit mass of irradiated material. The unit of absorbed dose is the Gray (Gy)

ALARA- Acronym for As Low as Reasonably Achievable.

Calibration- The check or correction of the accuracy of a measuring instrument to assure proper operational characteristics

Critical Organ- The organ or tissue, the irradiation of which will result in the greatest hazard to the health of the individual

Declared Pregnant Worker- A woman who has voluntarily informed her employer, in writing, of her pregnancy and the estimated date of conception

Dose Rate- The radiation dose delivered per unit of time

Dosimeter- A portable instrument for measuring and registering the total accumulated exposure to ionizing radiation

Gray- The international (SI) unit of absorbed dose in which the energy is equal to one Joule per kilogram

Half Value Layer- The thickness of any specified material necessary to reduce the intensity of an x-ray or gamma ray beam to one-half its original value

Inverse Square Law- The intensity of radiation at any distance from a point source varies inversely as the square root of that distance

Ionizing Radiation- Any radiation capable of displacing electrons from atoms or molecules, thus producing ions

Occupational Radiation Dose- The dose received by an individual in the course of employment.

Radio sensitivity- The relative susceptibility of cells, tissues, organs, organiHRH, or other substances to the injurious action of radiation

Sievert - The international (SI) of dose equivalent

Thermo luminescent Dosimeter (TLD)- Crystalline materials that emit light if they are heated after they have been exposed to radiation

Abbreviations

AERB	:	Atomic Energy Regulatory Board

BARC	:    	Bhabha Atomic Research Centre

ALARA	:   	As Low As Reasonably Achievable

RSO	:    	Radiation Safety Officer

Gy	:    	Gray

Pb	:    	Lead

TLD	:      	Thermo luminescent Dosimeter

NOC	:     	No Objection Certificate

SSD	:     	Source-Skin Distance

THF	:     	Tuned high frequency

Scope

Radiology Department

Responsibility

Quality/ Radiology staff / Management

Distribution

Radiology Department, Quality department

Process Details

The Department of Imaging Services assures accurate and reliable test result following the established standard operating procedure to facilitate better diagnosis and treatment of patients The Department also assures to protect the integrity and safety of its patient and staff.

The quality assurance programme is a comprehensive programme addressing equipment QA, Protocols, safety, education and surveillance.HRH Hospital will ensure to meet all the requirements of AERB.

Internal Quality Assurance:

HRH Hospital will ensure to have a peer review system in place to review the reports and outcomes of interventional procedures performed. This shall be done in structural manner, and the sample size, periodicity for each modality shall be defined. The result of such reviews shall be discussed with all stake holders in “discrepancy meetings” and the same shall be documented. The peer review can be performed by the head of department or by a group of peers, with or without blinding of the original reports. Discrepancies in the reports will be graded on the severity and impact on changes on patient management strategy, and the corrective and preventive actions taken to minimize these will be documented. The purpose is to prevent errors n future, and continuous quality improvement rather than computation or error rates of the individuals.

Verification and Validation of examination procedure

Verification of the examination procedure technically and clinically will be done by qualified, well trained radiologists on a daily basis

Work instructions shall be displayed on the work stations of the technicians

Verification of the examination procedures shall be strictly monitored by the radiologist

Validation of test results:

- Validation is done based on following

Daily control run and checked by Consultant Radiologist

Comparing the results with other potential organisations in the city

Participation in External Quality Control Programme.

All procedures and processes of laboratory are approved and monitored by radiologists

Inputs on clinical utility of the test results are obtained from consultants and analysed.

Surveillance

A routine quality control is monitoring and maintenance system incorporating state-of-the-art procedures should be established and conducted on a regular schedule. The maintenance program should include corrective maintenance to eliminate problems revealed by monitoring or other means before they have a serious deleterious impact on patient care.

The Department of diagnostic radiology facility should consider monitoring the following key components of the x-ray system.

Film-processing.

Basic performance characteristics of the x-ray unit.

For specialised Equipment

Examples of parameters of the above-named components and of more specialized equipment that may be monitored are as follows:

(a) For film processing:

An index of speed.

An index of contrast.

Base plus fog.

Solution temperatures.

Film artefact identification.

(b) For basic performance characteristics of the x-ray unit:

(1) For fluoroscopic x-ray units:

Tabletop exposure rates.

Centring alignment.

Collimation.

kVp accuracy and reproducibility.

mA accuracy and reproducibility.

Exposure time accuracy and reproducibility.

Reproducibility of x-ray output.

Focal spot size consistency.

Half-value layer.

Representative entrance skin exposures.

(2) For image-intensified systems:

Resolution.

Focusing.

Distortion.

Glare.

Low contrast performance.

Physical alignment of camera and collimating lens.

(3) For radiographic x-ray units:

Reproducibility of x-ray output.

Linearity and reproducibility of mA stations.

Reproducibility and accuracy of timer stations.

Reproducibility and accuracy of kVp stations.

Accuracy of source-to-film distance indicators.

Light/x-ray field congruence.

Half-value layer.

Focal spot size consistency.

Representative entrance skin exposures.

(c) For cassettes and grids:

(1) For cassettes:

Film/screen contact.

Screen condition.

Light leaks.

Artefact identification.

(2) For grids:

Alignment and focal distance.

Artefact identification

For specialized equipment:

For computerized tomography:

Precision (noise).

Contrast scale.

High and low contrast resolution.

Alignment.

Representative entrance skin exposures.

System in place to ensure the appropriateness of the investigations and procedures for the clinical indication

The investigation orders are screened prior to performing of the imaging or interventional procedure to ensure that they are appropriate investigation (as per current best practice guidelines and patient safety) based on for the clinical indication, otherwise alternate investigations are offered in consultation with the treating doctor.

Calibration and maintenance of equipments

X-Ray:

a.	Daily maintenance of the machine

b.	Checking processing chemical levels

c.	Maintenance of the cassettes.

C-Arm:

a.	Performing daily check-up of the Machine Functions

b.	Daily Maintenance of the machine

Corrective and Preventive maintenance

Preventive maintenance

Preventive maintenance should be performed on a regularly scheduled basis with the goal of preventing breakdowns due to equipment failing possible preventive maintenance procedures are:

Visual inspection of the mechanical and electrical characteristics of the x-ray system covering such things as:

checking conditions of cables,

assuring cleanliness with respect to spilling of contaminants in the  listening for unusual noises in the moving parts of the system

following the manufacturer’s recommended procedures for cleaning and maintenance of the equipment, and

Regular inspection and replacement of switches and parts that routinely wear out or fail.

Corrective maintenance

All the equipments shall be monitored under the Annual Maintenance Contract (AMC) on a quarterly. The service engineer shall do periodic calibration of the equipments and shall also be responsible for untimely breakdown of the instrument

Breakdown management:

a.	During breakdowns shutdown and restart the unit, check all Input & cables for loose connections. In case this fails, complaint should be logged into Instrument History Card and Work order should be raised and given to the Biomedical In charge mentioning the Machine Name, time of breakdown.

b.	It is the duty of the reporting person to place Machine OUT OF ORDER / DO NOT OPERATES Board near the equipment.

c.	The Biomedical engineer will inspect the machine & take necessary action as per their protocol.

d.	It is the duty of the Radiographer to inform Front desk, ER , ICU and NICU,  & Radiologists  the breakdown time and follow up on rectification till its working time every  12 Hours the status of the breakdown .

e.	In case of Major Breakdown the COO as well as the service engineer should be informed.

Corrective & preventive actions documentation

1.	Incident Report is raised for all Breakdowns more than 24 hours.

2.	After rectification service report is received and filed & the same is entered in Instrument History Card.

3.	All the AMC details of all the equipments of the lab shall be documented in a log book which shall include the following

Date of purchase

Name of manufacturer

Date of all preventive Maintenance Checks

Date of past breakdown check along with name and contact of the service engineer

4.	A sticker shall also be pasted n the equipment mentioning the date of last preventive maintenance check and also the contact of the Service Engineer

5.	A list of the individuals responsible for monitoring and maintenance techniques

Training

The program should include provisions for appropriate training for all personnel with quality assurance responsibilities. The training should be specific to the facility and the equipment. This should include both training provided before the quality assurance responsibilities are assumed and continuing education to keep the personnel up-to-date.

Practical experience with the techniques conducted under the supervision of experienced instructors, either in the facility or in a special program, is the most desirable type of training. The use of self-teaching materials can be an adequate substitute for supervised instruction, especially in continuing education programs, if supervised instruction is not available.

Review

The Quality Assurance Committee and/or the practitioner in charge should review the facility’s quality assurance program to determine if its effectiveness could be improved.

Items suggested for inclusion in the review include:

The reports of the monitoring and maintenance techniques to ensure that they are being performed on schedule and effectively. These reports should be reviewed at least quarterly.

The monitoring and maintenance techniques and their schedules to ensure that they continue to be appropriate and in step with the latest developments in quality assurance.

They should be made current at least annually.

The standards for image quality to ensure that they too are consistent with the state-of the-art and the needs and resources of the facility. These standards should be reviewed at least annually.

The results of the evaluations of the effectiveness of the quality assurance actions to determine whether changes need to be made. This determination should be made at least annually.

The quality assurance manual should also be reviewed at least annually to determine whether revision is needed.

Departmental coordinator shall be vigilant about the key characteristics. Based on the observation, departmental coordinator shall record his / her remark against the key characteristics as to whether the key characteristics meet the acceptance norms or not. Specific comments for the key characteristics may also be written.

| S. No. | Key Characteristics | Acceptance Norms / Criteria | Remark |
| --- | --- | --- | --- |
|  | Quality of X-ray image | 90 % of the X-ray shall be of good quality | Radiologist shall survey a sample of X-ray films to identify the %age of films which are of good quality |
|  | Functionality of equipments | All radiology equipments shall be in best functional condition |  |
|  | Timely intimation of critical results | Within ½ hour |  |
|  | Waiting time for investigation. | X ray : 30 mins or less (90% cases) |  |
|  | Wastage of film because of repeat process | >5% |  |
|  | TLD badges | Use of TLD badges shall be as per AERB guidelines |  |

External Quality Assurance

External Quality Assurance can be done by the following ways

Peer review- In this method all the leading surgeons and radiologist shall be called for to sit and discuss the various radiology cases

Regular Audits

Reviewing of reports from external agency